Ecotracer spatial contaminant concentration forcing drivers

Jan 5, 2017

Spatial and temporal contaminant concentrations can be driven in Ecotracer when running from Ecospace via a .csv text file. These files contain contaminant concentrations in an x, y, group and time stamp point format. Each record in the file represents a time stamped point observation of contaminant concentration for a group. Files can contain sparse data points in both space and time for different groups as each record in the file contains identifiers for space, time and group. The data in the file does not need to be continuous. If a driver file is loaded and Ecotracer is running from Ecospace the model will overwrite the predicted contaminant concentration with the value from the driver file based on the location, time stamp and group. The time stamp of the model time step is based on the model year set in the Ecopath > Model parameters form. If there is no overlap of time steps no data will be loaded from the file. To drive the environmental concentration layer, use the zero group index.

The contaminant concentration units in the file are t/km2 for the area of the cell. If you are driving the environmental contaminants this concentration may be different then the values entered as the Initial concentration (t/km2): on the Ecotracer inputs form. The value entered as the initial concentration is the average across the map. If you have restricted the initial environmental contaminant via the Ecospace > Inputs > Maps > Contaminates (rel) map, the contaminants will be concentrated into fewer cells using the cell weights entered in this map. This means if you have a 10 x 10 cell map and have concentrated the contaminates into a single point source cell the concentration in that cell will be 100 times higher than the value entered as the initial concentration. This allows you to enter the same initial concentration values for both the Ecosim (spatially average values) and Ecospace (spatially discrete values) models.

How to use it.

Contaminant concentration files and be loaded via the Contaminant concentration driver file button on Tools > Contaminant tracing > Ecotracer inputs form. Once a file is loaded it will stay in memory until a new model is loaded or the Clear button is clicked

Data validation.

Two forms of data validation are performed when loading, read validation and bounds validation.

Read validation.

For a record to be successfully read into memory it must pass the read validation: it must have the correct number of fields in the correct order, the date format must be in the international format of “yyyy-MM-dd”. If one of these fails the record will not be read from file.

Bounds validation.

Once a record is read into memory it will be check for: date is within the model run dates, row and column number are in bounds of the current map. If a record fails one of the bounds tests a waring message will be posted but the record will still be available during the run. This allows the user to adjust the model run or change scenarios to include the data.

Ecospace time series “Row, Col” input file format.

These files are a fixed format comma separated text file, with a header row containing column names.

These files can be opened, edited and saved directly from Excel in the .csv format. Care must be taken when saving from Excel to preserve the date format in “yyyy-MM-dd”

While the header row is ignored during the read it must be present in the file. The column order of the data is fixed and must be in the following order for the data to be read correctly.

The first two columns are the one based row and column numbers from the underlying Ecospace map, starting at the top left hand corner of the map. These data must be indexed to the currently loaded Ecospace base map. No attempt is made to spatially reference the data other than placing them by row and column index. If you have multiple Ecospace base maps, of different spatial resolutions, you must preprocess these data to match the correct row and column indexes.

Column three contains the one based Ecopath group index, this is the group number the data will be applied to. In Ecotracer the environmental concentrations are stored in the zero index, to drive these data use a zero in the group index column.

Column four contains the date of the record in the international date format of “yyyy-MM-dd”, for example 2000-12-01, 2001-01-01. The day portion of the time stamp is ignored but must be present as -01 for the date to be recognized correctly. If the date is in any other format it may not be recognized. If you are using Excel to format the data make sure you set the custom date format to “yyyy-MM-dd” before saving to .csv. Ecospace calculates the model time step date based on the Ecopath Model year, from the Ecopath Model parameters page, the number of time steps (/year), from the Ecospace parameters page, and the current model time step.

Column five contains the contaminant concentrations.

Example file format

Row,Col,GroupID,Date(yyyy-MM-dd),ObservedValue

5,5,1,2000-01-01,0.003981269

5,5,1,2000-02-01,0.00360784

5,5,1,2000-03-01,0.00352945

5,5,1,2000-04-01,0.003449183

5,5,1,2000-05-01,0.002551451

5,5,1,2000-06-01,0.002687412

5,5,1,2000-07-01,0.001394481

5,5,1,2000-08-01,0.001002992